ACE 방법과 가상계측기 정보를 이용한 공간변수의 비선형 추정방법

박문규, 이은기, 김용희

한국전력공사 전력연구원

Nonlinear Fitting of Spatially Distributed Variables using Virtual Detectors

based on ACE Algorithm

###### Moon Ghu Park , Eun Ki Lee, Yong Hee Kim

Korea Electric Power Research Institute, KEPCO

##### INTRODUCTION

The estimation or reconstruction of the unmeasured spatial variables has been widely applied in various scientific and engineering fields, especially for the monitoring systems of safety critical and expensive detectors are required. In this paper, a concept of Virtual Detector Method (VDM) is introduced to enhance the estimation accuracy of the spatially distributed variables without additional cost of detectors. Normally, one can reconstruct the continuous spatial variable by synthesizing the discrete measurements from spatially distributed detectors by Fourier, spline, and/or nonlinear fitting method like neural networks. However, it is not easy to get satisfactory accuracy with limited number of detectors in spatial domain. The objective of this paper is to give a constructive method of applying virtual detectors. The key factor of VDM is to justify the signal from virtual detectors. In this paper, the signal transferred to virtual detectors are extracted from the neighboring real detectors via the correlation specifying the accurate and robust relationships between them. Then the desired continuous form of the measurement can be synthesized via the fitting methods with enhanced accuracy with increased number of detectors.

##### ACE ALGORITHM

The optimal correlation between measured signal and virtual detector is extracted by using Alternating Conditional Expectation (ACE) algorithm.1,2,3 The ACE method is a generalized nonlinear regression algorithm that yields an optimal relationship between a dependent variable and multiple independent variables. By using the ACE algorithm, we can get transformations that produce the best-fitting additive model. Generally, this objective can be achieved by treating each value of the transformed dependent variable as the expectation of several realizations of the sum of transformed independent variables. Initially, it is not required to guess explicit functional forms of the transformations. This property is one of the remarkable advantages of the ACE algorithm. Once the optimal transformations are found by iteration, one can determine the coefficients of functional form for the transformed dependent and independent variables through the simple regression analysis.

By introducing the ACE algorithm, we can get considerable advantages over traditional nonlinear regression techniques;

1. The ACE algorithm guarantees the convergence of the transformations,

2. An initial selection of the functional forms is not required. Thus, independent of the order of the fitting function or the sizes of data sets, the ACE algorithm can be applied to get complex correlation between variables.

3. Iterative modifications of the functional relationships are not needed.

Real Detectors

Virtual Detectors

Spatial variation to be reconstructed

Fig. 1.    One-dimensional spatial detector system with 5-real/4virtual detectors.

Let's consider a problem to reconstruct the spatial variation as depicted in Fig. 1. The ACE algorithm is applied to find out the optimal correlation between virtual detector signal (dependent variable), Pv, and real detector signal (independent variables), {Di, i=1,_,5}. For the systems which have difficulties in extracting the optimal relationship between output y and input variables xi by conventional statistical methods, the transformation techniques can be generally applicable. For multivariate regression problem with a set of data {Pvi, D1i, D2i,_, D5i, i=1,_,N}, the optimal transformations of multivariate problems are readily derived by ACE algorithm as follows,

(1)

and

,                          (2)

where S[T|qi] means a conditional expectation at qi and is determined by evaluating an weighted expectation about T around the neighboring values in the interval [i-M, i+M] for a given user defined value, M. All transformations should be mean zero functions. The transformations of each variables are coupled each other and solved by alternative-iteration procedure to minimize the square error of regression,

(3)

There are several weighting or smooth techniques such as Histogram, Nearest neighber, kernel, regression, and supersmoother. In this paper, kernel method is used with weighting function defined on the real line with maximum at z = 0.

Basically the ACE algorithm produces the discrete optimal transformtions for given discrete data sets. Therefore, after the optimal transformations are calculated, we construct a simple regression model based on the least square method to obtain the continuous polynomial functions describing each transformation. To consider the extrapolation of data, each transformation was devided into 3 regions.

##### SYNTHESIS OF THE DISCRETE MEASUREMENTS

The ACE algorithm is introduced to construct the signal at virtual detectors by setting up optimal correlation between real and virtual detectors at each spatial location. Analytic function corresponding to each optimal transformation was calculated by simple regression model. Finally, both signals from real and virtual detectors are synthesized to get more accurate measurements. We demonstrated the synthesis method using Fourier fitting method (FFM) compared with reference data set.

The key feature of VDM is how to get the optimal correlation between real 5    detectors and each of 9 virtual detector information. After the 9 detector information is determined by optimal correlation, output variable is computed by using 5 points FFM. In FFM, the following equation is used to generate axial powers;

(4)

where ND=no. of detectors, n = no. of mode (n=1,_,ND), an(bn) = Fourier coefficients to be determined with 5 detectors , and Bc = fitting parameters minimizing the average RMS error. The Fourier coefficients are determined by following equations;

,    (m= 1,..., ND),                                    (5)

where PDm, Zl and Zh mean the detector signal, relative lower and upper height at the m-th detector level, respectively.

##### NUMERICAL RESULTS

The application to a collection of massive data set shows the apparent improvements in mean, root mean square, and maximum estimation errors. The concept of virtual detector can be a viable approach to enhance the estimation accuracy of the spatially distributed variables without additional detector cost. To verify the usefulness of the method, we have reconstructed a total of 21,000 cases of one-dimensional axial profile fitted by using 5 real detectors and 4 virtual detectors located between each real detectors.

Fig. 2 shows an example of relationships between 8th virtual detector information and each normalized 5 detectors. Fig. 2 shows that it is very difficult to find initial trial functions due to the diffuse and complex nature of the relationship between real and virtual. Fig. 3 shows the avg. absolute RMS error of VDM. The avg. error line of 5-FFM is ~0.03 while ~0.01 of VDM with 9-FFM.

Fig. 4 shows the result for a sample profile indicating the problem of current 5-FFM for which profile gives some fluctuating signal due to sinusoidal basis function with 5-FFM. This phenomenon is certainly improved with 9-FFM based VDM.

##### REFERENCES

1 L. Breiman and J. H. Friedman, Journal of the American Statistical Association, Theory and Method, 80, 580 (1985).

2 E. K. Lee, Y. H. Kim, K. H. Cha, and M. G. Park, Proc. KNS Autumn Meeting, Seoul, Korea, October (1998)

3 E. K. Lee, Y. H. Kim, K. H. Cha, and M. G. Park, Annals of Nuclear Energy, 26, 983 (1999)

Fig. 2    Correlation of sample data obtained by ACE algorithm

Fig. 3    Avg. absolute RMS error of VDM for 9-FFM and 5-FFM

Fig. 4    Sample profile of reconstructed variable 9-FFM and 5-FFM