# 불안정 공정에 대한 비례-적분-미분 제어

## 박 진현,성 수환,이 인범

## 포항공대 화학공학과, 환경공학부

# PID Control Strategy for Unstable Processes

## Jin Hyun Park, Su Whan Sung, and In-Beum Lee

## Department of Chemical Engineering, School of Environmental Engineering, POSTECH

### Introduction

The PID controller has been widely used in process control industry because of its simple structure, robustness to the modeling error, easiness in operation and availability of on-line tuning rules. Åström and Hägglund (1984) proposed a relay feedback method to get the ultimate frequency and the ultimate gain. Sung et al. (1995) suggested a modified relay feedback method to get more accurate ultimate information of the process. Park et al. (1997) suggested an improved relay feedback method to obtain the ultimate information with static load disturbance and also pointed out how to get the static gain of the process and the ultimate data sets simultaneously. Above methods and other many identification methods and corresponding tuning rules are available to tune the PID controller for stable processes.

However, it is very difficult to control unstable processes with time delay. Even though two tuning rules (Venkatashankar and Chidambaram 1994, De Paor and O’Malley 1989) have been presented, they show a poor closed-loop response. Recently Jacob and Chidambaram(1996) suggested three design procedures using model reference method, synthesis method and IMC method. The controller by model reference method shows a good response by choosing an appropriate damping factor and a suitable settling time. However, it needs repetitive tuning procedures since the user has to specify an appropriate damping factor and a suitable settling time by trial and error.

Therefore, we propose an enhanced PID control strategy to incorporate unstable processes more efficiently and systematically using two feedback loop design method.

### Model Parameter Identification and Control Strategy

We introduced the improved relay feedback method (Park et al., 1997) to identify the ultimate information and the static gain of the process using only one relay test. The unstable process transfer function can be modeled as the following unstable FOPTD model

Gm(s)=kmexp(−θms)τms−1	(1)

From the ultimate data sets and process gain obtained from the improved relay feedback, we can easily find the unstable FOPTD model parameters as follows

τm=kmkcu2−1ωu2	(2)

θm=tan−1(τmωu)ωu	(3)

The block diagram of the proposed method is shown in Fig. 1. Here, the P controller (kci) in the inner feedback loop plays an important role in changing the open-loop unstable process (Gp(s)) to an open-loop stable process (G(s)).

Let’s consider the following unstable FOPTD process.

Gp(s)=kexp(−θs)τs−1	(4)

From the Nyquist stability criterion, we can find out the range of the kci which can stabilize the unstable FOPTD process. The frequency response of the open-loop transfer function of the inner feedback loop in Fig. 1 can be written easily.

GOL(jω)=−kkcicos(θω)+τωsin(θω)−j[sin(θω)−τωcos(θω)]1+(τω)2	(5)

In order to stabilize the unstable FOPTD process the following properties should be satisfied. First, at  = 0 the following should be met.

GOL(0)=−kkci<−1	(6)

1k<kci	(7)

Second, the imaginary part of GOL(ju) is zero and the real part of GOL(ju) has to be larger than -1 at the ultimate frequency (u).

sin(θωu)=τωucos(θωu)	(8)

kci<1k1+(τωu)2=1kτθθωusin(θωu)	(9)

Since the minimum value of θωusin(θωu) is 1, must be larger than .

From (7) and (9), the boundary of the P controller gain which can stabilize the unstable FOPTD process is obtained.

kmin=1k<kci<1k1+(τωu)2=kmax	(10)

Then, we can choose the following as a design value of kci .

kci=kminkmax=1k41+(τωu)2	(11)

Since Gp(s) is identified as the unstable FOPTD model (Gm(s)), the closed-loop system transfer function (G(s)) containing the inner feedback loop is as follows

Gs≃GMs=Gms1+kciGms=kmexp(−θms)τms−1+kmkciexp(−θms)	(12)

And (12) can be transformed to the stable second-order plus time delay (SOPTD) model using model reduction technique.

Let’s consider the following reduced SOPTD model.

Grms=krmexp(−θrms)τrm2s2+2τrmζrms+1	(13)

Here, the static gain (krm) of the reduced model (Grm(s)) is determined from (14)

krm=GM(0)=kmkmkci−1	(14)

The amplitude ratios of GM(s) and the reduced model (Grm(s)) are given by

GM(jω)=kmkmkcicos(θmω)-12+τmω−kmkcisin(θmω)2	(15)

Grmjω=krmexp(−θrms)τrm2s2+2τrmζrms+1s=jω=krm(1−τrm2ω2)2+(2τrmζrmω)2	(16)

where, m and m are estimated to satisfy the following equation

minτrm,ζrmGM(jω)−Grm(jω)2	(17)

Using a least-square method,m and m fulfilling (17) can be directly estimated from (18) and (19) (Levy 1959, Sung and Lee 1996).

τrm4GMjωi2ωi4+(4τrm2ζrm2−2τrm2)ωi2GMjωi2=krm2−GMjωi2	(18)

0<ω0<ω1< ⋯ <ωi< ⋯ ≤ωu	(19)

and also the time delay (rm) of the reduced model can be obtained using (20).

θrm=π+arctan2(−2τrmζrmωu, 1−τrm2ωu2)ωu	(20)

Here, u represents the ultimate frequency of GM(s) and i`s are located with equal space between 0 and u. We consider the frequency below u in equation (12) and (13) since the controller operates in this frequency range.

Since the unstable process is stabilized with the P controller in the inner feedback loop, we can use the PID controller to control the stabilized process (G(s)). And G(s) in Fig. 1 is modeled as the SOPTD models using model reduction technique, the PID controller tuning parameters are immediately calculated from the ITAE tuning rule (Sung et al. 1996).

### Simulation Study

Let us consider the following process transfer function

Gp(s)=4.0exp(−2s)(4s−1)	(21)

By carrying out the improved relay feedback method we get km = 4.010, kcu = 0.491 and wu = 0.508, and using (2) and (3) we obtain tm = 3.325, qm = 2.040. From (11) kci is decided 0.350. For the set point change, kc = 0.068, ti = 1.885 and td = 4.296 are calculated from the ITAE tuning rule (1996, Sung et al.).

The control performance of the proposed method for the set point change is compared with the results of De Paor and O’Malley (1989), Jacob and Chidambaram’s synthesis method and model reference method (1996) as shown in Fig. 2

### Conclusions

We proposed an enhanced PID control strategy including an inner feedback loop and the corresponding simple tuning relation. The unstable first-order plus time delay model is obtained from the improved relay feedback method and the inner feedback loop plays an important role in stabilizing unstable process. Model reduction technique is used to get a stable second order plus time delay model and then, the outer loop PID controller can be tuned from the ITAE tuning rule. The proposed control strategy is very simple and shows superior control performance compared with the previous methods.

### Literatures Cited

Åström, K.J. and Hägglund, T., Automatica, 20, 645(1984)

De Paor, A. M., and O’Malley, M., Int. J. Control, 49, 1273(1989)

Jacob, E. F., and Chidambaram, M., Comp. & Chem. Eng., 20, 579(1996)

Levy, E. C., IRE Tran.AC, AC-4, 37(1959)

Park, J. H., Sung, S. W., and Lee, I., Automatica, 33, 711(1997)

Sung, S. W. and Lee, I., Ind. Eng. Chem. Res., 35, 2596(1996)

Sung, S. W., O, J., Lee, J., Yu, S., and Lee, I., J. Chem. Eng. Jpn., 29, 990(1996)

Sung, S. W., Park, J. H, and Lee, I, Ind. Eng. Chem. Res., 34, 4133(1995)

Venkatashankar, V. and Chidambaram, M., Int. J. Control, 60, 137(1994)

### Acknowledgment

This work was supported in part by Korea Science and Engineering Foundation (KOSEF) through the Automation Research Center at Pohang University of Science and Technology.

Figure 1. Proposed control algorithm block diagram for unstable process

Fig. 2 Closed-loop response for unit step change in the set point