상압하에서 Water+1,3-Propanediol, Water+1,3-Propanediol+LiBr 계의 기액상평형

문성용, 김진수, 이흔

한국과학기술원 화학공학과

Vapor-Liquid Equilibrium at 101.3 kPa for Water+1,3-Propanediol

and Water+1,3-Propanediol+LiBr

Song-Yong Mun, Jin-Soo Kim, Huen Lee

Dept. of Chem. Eng. KAIST

## Introduction

Investigation concerned with the effect of salt addition on the vapor-liquid equilibria of binary systems are of industrial importance. In the air-cooled absorption chiller operation, the information on the VLE data of the system including LiBr can be helpful in developing a new working fluid.

The present work studies the VLE for the systems of water+1,3-propanediol and water+1,3-propanediol+LiBr with varying concentration of salt. The addition of LiBr to this solvent mixture increases the amount of 1,3-propanediol in the vapor phase at equilibrium. This indicates a preferential association of LiBr with water.

For the salt-free system, the correlations by activity coefficient models such as Wilson(1964), NRTL(1968), UNIQUAC(1975) equations were carried out, and the extended UNIQUAC model of Sander et al.(1986) was applied to the calculations of water+1,3-propanediol+LiBr VLE data.

# Experimenatal

Chemicals.  The chemicals used were distilled water, 1,3-propanediol(Aldrich,98%), LiBr(Aldrich, 99+%). All chemicals were used without further purification.

Procedure.  The recirculation still used is shown in Figure 1. Liquid sample, about 230mL, was fed into equilibrium vessel A through port L. Energy was supplied from silicone oil(Dow corning, 200®fluid,550cs) heated by electrical heater H. By increasing the temperature of silicone oil gradually, the superheat of either liquid or vapor phase may be prevented.

When sufficient time had elapsed and the system was thought to have achieved equilibrium, a sample was taken in a very short time in each phase. To prevent partial condensation, the vapor jacket was wrapped with an electrical heating tape controlled by a variable power source. The stirring bar in equilibrium vessel increase the effect of mixing in the salt-containing solution .

Method of analysis.  The compositions of the vapor phase condensate samples are determined from measurements of their refractive indexes at 35℃ with an Abbe-type refractometer. Since the liquid phase contained a salt, the salt was separated using an evaporating apparatus. The liquid phase composition other than the salt(salt-free basis) was analyzed by a refractometer. And the mole fraction of salt in this liquid phase was obtained from the mass of the separated salt.

The accuracies in the calibrations were within ±0.005 mole fractions. The temperature in the equilibrium vessel was measured with an accuracy of ΔT=±0.05K.

# Results and Discussion

Experimental data.  The apparatus was checked by comparisons of vapor-liquid equilibrium measurements for ethanol +water with literature values at 101.3 kPa. Obseved data had satisfactory agreement with data from the references[1].

Furthermore, vapor-liquid equilibrium for water(1)+1,3-propanediol(2) was measured at 101.3kPa. These results was shown in Figure 2. A thermodynamic consistency test was applied to the experimental data by using the point test of Fredenslund et al.(1971) and the area test of Herington(1951). The result of point test was 0.004 when the order of Legendre polynomial was 5. Thermodynamic consistency was also confirmed in the Herington’s test.

Isobaric VLE data at 101.3kPa were measured for the system water(1)+1,3-propanediol(2)+LiBr(3), and the results are presented in Table 2. The liquid concentration is presented on a salt-free basis x'(x'=n1(n1+n2) where n1and n2 represent the number of moles of solvents 1and 2). This makes it easy to investigate the effect of salt presence on the VLE behavior of the salt-free system. In this work, the presence of salt increases the relative volatility of 1,3-propanediol.

When a salt is dissolved in homogeneous liquid mixture, it will form liquid-phase associations or complexes with one species in the liquid more than the other. This interaction will reduce the activity of the more attracted component and increase the activity of the less attracted component. Hence, the less attracted component is expelled or salted out from the liquid solution. In our case, the 1,3-propanediol molecules are less attracted to salt ions and tend to go into the vapor phase.

Calculation of Phase equilibrium.  Experimental data for water(1)+ 1,3-propanediol(2) were correlated using Wilson(1964), NRTL(1968) and the UNIQUAC(1975) models for liquid-phase activity coefficients. The bubble point ,T, and vapor composition ,y, were calculated from the models. The vapor pressures of pure solvents were determined from the equations given in the references[2]. Model parameter values, mean absolute deviations in vapor phase composition (Δy) and bubble point temperature(ΔT) are presented in Table 1.

In the case of a mixed solvent-salt system, the results were correlated using the extended UNIQUAC activity coefficient model of Sander et al[3]. The model parameters are ion specific ones, and no ternary parameters are required. The results of the calculations using Sander’s model are shown in Figures 3 and 4. The model represented experimental data with good accuracy, especially at a low salt concentration. The mean absolute deviation in vapor composition was 0.013 and the corresponding error in bubble point was 0.70 K. The deviations in the calculated values were found to be rather high at high salt concentration. The UNIQUAC volume(rk) and surface area(qk) parameters are taken from the original paper of Sander et al. (1986). Fortunately, Li+-H2O, H2O-Li+, Br-H2O, H2O-Br interaction parameters are existed in the Sander’s paper. Moreover, the solvent-solvent parameters were given by the salt-free system correlation. The other model parameters were estimated by minimization of the following objective function:

F=N[(lnγ1,exptl−lnγ1,calcd)2+(lnγ2,exptl−lnγ2,calcd)2]

where γexptland γcalcd refer to the experimental and calculated activity coefficients, respectively, and Nis the number of data points. The calculated and experimental activity coefficients were obtained assuming that vapor phase is ideal. Since the pressure is atmospheric and considering other sources of uncertainties in the experimental measurements, this assumption is acceptable. The UNIQUAC reference interaction parameters akl  and the δij,mparameters are presented in Tables 3 and 4.

## Conclusion

The experimental VLE data at 101.3 kPa for the system of water+1,3-propanediol and water+1,3-propanediol+LiBr were measured. The salt investigated in this work exhibited a salting-out effect on 1,3-propanediol. This salt effect increases as the salt concentration increases.

Experimental data of the salt-free system are very well correlated using the Wilson, NRTL, UNIQUAC models. Model parameters are obtained and used to calculate the VLE for the salt system. The extended UNIQUAC model of Sander et al. was found to correlate the experimental data of the salt system with reasonable accuracy.

## References

1. Gemhling, J., Onken, U., Arlt, W. Vapor-Liquid Equilibrium Data Collection; DECHEMA: Germany, 1981

2. Dauber, T. D., Danner, R. P. Physical and Thermodynamic Properties of Pure Chemicals: Data Compilation; Hemisphere Publishing Co.: New York, 1989

3. Sander, Bo., Fredenslund, A., Rasmussen, P. Calculation of vapor-liquid equilibrium in mixed solvent/salt systems using an extended UNIQUAC equation. Chem. Eng. Sci. 1986, 41, 1171-1183.

Table 1. Results of the Correlation with Wilson, NRTL, and UNIQUAC Models for water(1)+1,3-propanediol(2)

| Model | parametersa | T | y |
| --- | --- | --- | --- |
| Wilson | Δλ12= 2651.40,Δλ21= 744.92 | 0.17 | 0.006 |
| NRTL | Δg12= 7552.0, Δg21 = -3640.6, α= 0.3 | 0.70 | 0.005 |
| UNIQUAC | Δμ12 = -401.56, Δμ21 = 1175.0 | 0.40 | 0.004 |

a All energy parameters are expressed in Jmol-1

Table 2. Experimental VLE data for water(1)+1,3-propanediol(2)+LiBr(3) at 101.3kPa

| T(℃) | x1' | x3 | y1 | T(℃) | x1' | x3 | y1 | T(℃) | x1' | x3 | y1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 118.0 | 0.484 | 0.007 | 0.983 | 176.2 | 0.090 | 0.027 | 0.765 | 128.1 | 0.466 | 0.060 | 0.972 |
| 116.2 | 0.520 | 0.008 | 0.986 | 126.1 | 0.420 | 0.029 | 0.968 | 132.1 | 0.416 | 0.061 | 0.962 |
| 126.2 | 0.372 | 0.008 | 0.970 | 158.2 | 0.163 | 0.031 | 0.879 | 125.9 | 0.504 | 0.064 | 0.976 |
| 130.1 | 0.330 | 0.009 | 0.962 | 107.2 | 0.810 | 0.028 | 0.996 | 120.5 | 0.603 | 0.066 | 0.986 |
| 123.3 | 0.419 | 0.012 | 0.973 | 145.0 | 0.240 | 0.031 | 0.927 | 114.3 | 0.743 | 0.068 | 0.994 |
| 143.7 | 0.221 | 0.010 | 0.932 | 113.6 | 0.643 | 0.030 | 0.992 | 168.1 | 0.160 | 0.069 | 0.865 |
| 136.0 | 0.280 | 0.011 | 0.949 | 110.4 | 0.729 | 0.032 | 0.994 | 118.0 | 0.667 | 0.073 | 0.992 |
| 112.0 | 0.627 | 0.014 | 0.992 | 107.7 | 0.820 | 0.036 | 0.995 | 156.2 | 0.230 | 0.078 | 0.905 |
| 170.0 | 0.101 | 0.015 | 0.797 | 165.1 | 0.140 | 0.039 | 0.858 | 134.0 | 0.450 | 0.090 | 0.963 |
| 109.6 | 0.702 | 0.016 | 0.994 | 109.4 | 0.781 | 0.038 | 0.994 | 150.1 | 0.297 | 0.092 | 0.926 |
| 158.3 | 0.152 | 0.019 | 0.881 | 170.3 | 0.121 | 0.039 | 0.831 | 132.7 | 0.479 | 0.094 | 0.965 |
| 107.9 | 0.773 | 0.024 | 0.995 | 106.3 | 0.880 | 0.039 | 0.998 | 144.1 | 0.353 | 0.093 | 0.944 |
| 135.5 | 0.308 | 0.026 | 0.950 | 153.2 | 0.219 | 0.055 | 0.912 | 199.5 | 0.065 | 0.118 | 0.538 |
| 152.1 | 0.191 | 0.028 | 0.902 | 147.8 | 0.259 | 0.057 | 0.931 | 210.1 | 0.048 | 0.142 | 0.408 |
| 115.7 | 0.585 | 0.027 | 0.988 | 157.1 | 0.203 | 0.059 | 0.903 | 214.8 | 0.030 | 0.168 | 0.267 |

Table 3. UNIQUAC Reference Interaction Parameters (akl/K)

|  | H2O | C3H8O2 | Li+ | Br- |
| --- | --- | --- | --- | --- |
| H2O | 0.0 | -48.3a | -59.2b | -43.5b |
| C3H8O2 | 141.33a | 0.0 | 1756.73a | 1084.64a |
| Li+ | -484.2b | 885.93a | 0.0 | 10.0a |
| Br- | -128.8b | 868.96a | -903.96a | 0.0 |

Table 4. δij,mParameters(K) (δij,m=δji,m)

|  | m=H2O | m= C3H8O2 |
| --- | --- | --- |
| LiBr | 35098.93a | 197995.8a |

a Estimated from the data of this work.

b Sander et al.(1986)

Fig3. The comparison of vapor composition       Fig4. The comparison of bubble point