Gas assisted injection molding의 fan-shaped 형상에서의

유체흐름의 방향

임 광희, 서 영수

대구대학교 화학공학과

Flow directions under fan-shaped geometry

in gas assisted injection molding

Kwang-Hee Lim and Young Soo Soh

Department of Chemical Engineering

Taegu University

## INTRODUCTION

To design molds in such a way that the gas cores out all the thick sections and not the thin walls, one needs to predict the preferred direction of gas for a given geometry. The understanding of rules governing the preferred direction of gas is important for trouble shooting during mold try outs as well as in design stage.

A method to relate the preferred gas direction with the process variables was introduced in our previous work.1,2  The method requires a knowledge on the relations between resistance for the gas flow and processes variables such as resin flow length, cross section area of cavity, melt temperature, and existence of short shot.

In this paper, a new equation to describe the pressure drop requirement for a steady state-fan-shaped flow and is applied to the flows under various fan-shaped geometries.  In near future the experiment with various fan-shaped molds shall be performed to demonstrate the gas direction of gas assisted injection molding and the result of the experiment shall be compared with the diagnosis on the gas flow direction in gas assisted molding using suggested rule of thumb herein presented.

# THEORY

For incompressible fluids, continuity equation in cylindrical coordinates becomes :

1r∂∂ r(r vr)+∂ vz∂ z=0                                            (1)

when vθ is assumed to be zero velocity.

Momentum equation for Newtonian fluid becomes:

ρ(∂ vr∂ t+vr∂ vr∂ r+vz∂ vr∂ z)=−∂ P∂ r+μ[∂∂ r(1r∂∂ r(r vr))+1r2∂2vr∂ θ2+∂2vr∂ z2]

ρ(∂ vz∂ t+vr∂ vz∂ r+vz∂ vz∂ z)=−∂ P∂ z+μ[1r∂∂ r(r∂ vz∂ r)+1r2∂2 vz∂θ2+∂2 vz∂z2]                                                                  (2)

In order to compare the order of magnitude of each term of Eq. (1) & (2), one may make these equations dimensionless with appropriate characteristic variables.

When  (HR0)21θ2<<1  and (HR0)2 is the same order as or less than 0 (1), neglecting end effect on both sides in  direction, the behavior of the flow between two fan-shaped plates in conventional injection molding may be considered as the part (i.e.,θ2π) of the radial flow between two entire round plates.

In addition, for ρvrHμ(HR0)<<1  and (HRO)2 << 1, Eq. (2) may be reduced into quasi-steady state equations except for the case of R1<<R0 of which the criterion of validity and applicability will be discussed later.

## Then the pressure distribution becomes

P=P1−P0ln R1R0lnrR0+P0                                           (3)

where P1 and P0 is the pressure at r= R1 and R0 respectively.

## The velocity profile is

vr(r, z)=h22μrP1−P0ln R1R0(z2h2-1)                                     (4)

if θ >>HR0, ρvrHμ(HR0)<<1 and (HR0)2<< 1.

Integrating vr(r,z) from Eq. (4) with z, the expression of the flow rate of melt phase (Q) of Eq. (5) is obtained as below.

Q=θ rHvr=2  vr(r, z)θ r dz=23θh3μP1−P0ln R0R1                (5)

where < vr >: Average velocity of melt phase flow

## DISCUSSION

As noted, the purpose of this paper is to qualitatively diagnose the instantaneous flow under fan-shaped geometry.  For diagnosing qualitatively the direction of the flow between two fan-shaped plates, all that one must do is to replace the presumed flow rate between entirely round two plates (i.e.,2πθQ ) with the real melt phase flow rate (Q) between two fan-shaped plates with angle of θ.

ΔPfan-plates=12μQH3θlnRoR1                                           (6)

if θ >>HR0, ρvrHμ(HR0)<<1 and (HR0)2<< 1 except for the case of R1<<R0 of which the criterion of validity and applicability is not shown in this paper.

Unlike the flow in a pipe, neglecting the amount of solidified polymer near relatively cold surface of mold, the melt phase flow between fan shaped plates results in shorter distance between leading front of melt polymer and rear gas-liquid interface, than that between initial R1 and R0 by geometric reason, which makes the initially predicted larger flow rate more dominating over the smaller one.  In case of such a heterogeneous geometry as the combination of both pipe flow and the flow between fan-shaped plates, one may conjecture that once the initial direction of most gas that entered is determined to between fan shaped plates, overall gas flow direction is determined to between fan shaped plates.

## NOMENCLATURE

Vr   	 velocity in r direction

VZ     	velocity in z direction

ρ    	density of polymer melt phase

θ  	the angle of the fan-shaped radial flow

R1	radius of the entrance for melt phase or gas injection

R0	radius of leading front of melt phase

h	distance between top or bottom fan-shaped plate and center–line of the cavity

H	distance between two fan-shaped plates

μ	Newtonian viscosity

Vr>	average radial velocity

λc	time constant

m, n	power-law indices

γ	shear rate

P1	pressure at r =R1

P0	pressure at r =R0

Q       flow rate of polymer melt phase

## ACKNOWLEDGEMENT

This research was supported (in part) by the Taegu University Research Grant, 1998.

REFERENCE

- Y. S. Soh and C.H. Chung, SPE ANTEC Tech. Papers, 43, 603 (1997).

- Y. S. Soh and C.H. Chung, Journal of Reinforced Plastics and Composites, to be published.(1998).