Equations of State for Hard-Sphere Chains:

## Attractive Contribution의 고려

김인하, 배영찬

한양대학교 응용화학공학부 분자열역학 연구실

Equations of State for Hard-Sphere Chains:

## Consideration of Attractive Contribution

In Ha Kim, Young Chan Bae

Division of Chemical Engineering and Molecular Thermodynamics Lab.

Hanyang University, Seoul 133-791, Korea

# Introduction

Among several categories of equation of state, the hard-sphere equation of state is adapted as a very useful reference system to represent numerous types of molecular fluids despite of its simplicity. Further, the hard-sphere system has another merit that it can be extended directly to the hard-sphere-chain system by introducing chain connectivity term, which is used in most existing equations of state for chain-like molecules.

In this study, we developed three hard-sphere-chain equations of state based on the well-known Carnahan-Starling hard-sphere equation differing in the perturbation terms, that have more simplified form than those of the models mentioned above. Each model is directly extended from the Carnahan-Starling equation by introducing a bonding term which accounts for the chain connectivity. We then compared each model with computer simulation data and experimental vapor-liquid equilibrium data of pure saturated fluids.

Theoretical Consideration

# Reference terms

The equation of state for hard spheres and hard-sphere chains generally consists of two contributions

Z=Zref+Zpert                           (1)

In this study, we employ the well-known Carnahan-Starling (C-S) equation of state as the reference equation for hard spheres.

ZCS=1+η+η2−η31−η3                         (2)

where η is defined as the packing fraction or reduced density which can be expressed as

η=πσ36ρ                              (3)

σ is the hard-sphere diameter and ρ=N/V is the number density.

In recent publications, two different types of bonding terms are found and adapted for this study.

Z1chain=1−rg(d+)−1                      (4)

Z2chain=1−rρ∂lng(d+)∂ρ                     (5)

where g(d+) is the pair radial distribution function for hard sphere chains at contact:

g(d+)=1−0.5η1−η3                         (6)

ρ∂lng(d+)∂ρ=η5−2η2−η1−η                    (7)

We employ eq.(5) to generate the equations for the hard-sphere chains from C-S equation by the following relationship proposed by Kim and Bae.

ZHSC=1+rZCS−1+Zchain                 (8)

where ZHSC stands for the reference equation representing the compressibility factor for hard sphere chains, Zchain is the bonding term which accounts for chain connectivity, and ZCS represents the C-S hard-sphere equation of state.

The final expression for the reference equation hard-sphere chain has the form of

Zref=1+r1+η+η2−η31−η3−1+1−rρ∂lng∂ρ             (9)

# Perturbation terms

### Simplest type of van der Waals attractive term

In 1873, van der Waals proposed the well-known equation of state for normal fluids

PvRT=vv−b−aRTv                          (10)

where P is the pressure, T is the absolute temperature, v is the molar volume, and R is the universal gas constant. Song et al. adapted this perturbation term and proposed the perturbed hard-sphere-chain (PHSC) equation of state to describe the phase equilibrium properties of fluids including chain-like molecules.

Zpert=−r2aρkT                            (11)

In eq.(10), parameter a which  reflects the attractive forces between two non-bonded segments, is temperature-dependent function given by

aT=2π3σ3εFakT/ε                       (12)

where

FakT/ε=1.8681exp−0.0619kT/ε+0.6715exp−1.7317kT/ε3/2       (13)

Perturbation term proposed by Bokis et al.

Bokis et al. pointed out this shortcoming that frequently occurs in most of existing equations of stateand suggested a simple expression for the perturbation term by directly correlating the Monte Carlo simulation data.

Zpert=q+qbηZpert1                       (14)

where Zpert1 is the attractive contribution to the compressibility factor for a spherical molecule (monomer).

Zpert=−q+qbηaρkT                        (15)

qa and qb are constants for each chain and vary approximately linearly with chain length according to

qa=0.856+0.144r                        (16a)

qb=2.54r−1                          (16b)

Square-well based equation for the attractive term

In 1998, Sadowski derived an equation of state using a physically meaningful expression while retaining a simplicity. For spherical fluids (r=1), the attractive contribution is described using a simple mean-field expression

Zpert1=−aTρkT                           (17)

The attraction parameters aT is related to the second virial coefficient BT and the van der Waals covolume b by

a(T)kT=b−B                            (18)

with bT=2π/3σ3 and B=2π/3σ031+(λ3−1)1−expu0/kT1+1/T.

In this model, the hard-sphere diameter σ is considered as a function of temperature following the Barker-Henderson theory:

σ=σ01−Cexp−3u0/kT                     (19)

with C=0.25.

For chain fluids, the compressibility factor of a chain fluid in the attraction term is calculated from that of a spherical fluid using the following relation

Zpert=Nr−rZpert1                          (20)

where Nr−r is the number of segment-segment interactions in the chain system. For a system which consists of spherical molecules only, Nr−r=N1−1 can be calculated as below

N1−1=C2=(2r)!2!(2r−2)!=r(2r−1)                  (21)

To estimate the number of interactions for the chain system, the ratio of Nr−r/N1−1 is approximated to be equal to the ratio of the radial distribution function:

Nr−rN1−1=gr(σ+)g1(σ+)                           (22)

The value g1(σ+) for the system of hard-spheres can be obtained from the Percus-Yevick equation

g1(σ+)=2+η2(1−η)3                         (23)

For hard-sphere chains, Chiew developed an analytical expression for the radial distribuion function:

gr(σ+)=2+(3r−2)η2r(1−η)3                        (24)

Eq.(24) reduces to eq.(23) for the hard-sphere system (r=1).

Using eq.(21) through eq.(24), a number of interactions in the chain system are now determined as:

Nr−r=ZpertZpert1=(2r−1)2+(3r−2)η2+η                (25)

Eq.(25) represents that Nr−r depends on both the segment number and the system density.

#### Results and Conclusion

Three characteristic parameters for each model are determined from experimental data of pure fluids. In comparison with computer simulation data, Bokis’ and Sadowski’s models show attractive contributions to the compressibility factor fairly well, however, the Song’s model cannot represent density dependence of the ratio of attractive compressibility factor of hard-sphere chain to that of spherical molecule when plotted versus the reduced density. Vapor-liquid equilibria of saturated pure fluids is reproducible using three characteristic parameters without any additional adjustable parameters with high accuracy. The Sadowski’s model best describes experimental vapor-liquid equilibria of pure hydrocarbons, while the Bokis’ model shows deviations in low-density region. To describe vapor-liquid equilibria of saturated pure water that shows a unique behavior, an additional scaling parameter, ζKB is introduced into the Sadowski’s model. The improved equation describes the vapor-liquid equilibria of water remarkably well while the original model shows a large deviation from the experimental data.

# References

- J. M. Prausnitz, R. N. Lichtenthaler, and E. G. D. Azevedo, Molecular Thermodynamics of Fluid Phase Equilibria, Prentice-Hall, Englewood Cliffs, NJ, 3rd Edition (1999)

- Y. Song, S. M. Lambert, and J. M. Prausnitz, Macromolecules 27 (1994) 441

- I. H. Kim and Y. C. Bae, Fluid Phase Equilibria 167 (2000) 187

- Chapman, W. G., Jackson, G., and Gubbins, K. E. Mol. Phys. 1988, 65, 1057

- C. P. Bokis, M. D. Donohue, and C. K. Hall, Ind. Eng. Chem. Res. 33 (1994) 1290

- G. Sadowski, Fluid Phase Equilibria 149 (1998) 75

FIGURE 1 Monte Carlo simulation data for the system of spherical molecule (r=1) which represents the compressibility factor. The solid line is calculated by the C-S equation. The open squares are simulation data reported by Barker and Henderson

FIGURE 2 Vapor-liquid equilibrium experimental data for saturated water. The lines are calculated by Sadowski’s model with different values of ζKB as denoted in the figure. The open circles are experimental data reported by Smith and Srivastava